[RETYM LOGO]

Retym, Inc.

HYP Gen1 CDR DAC
Specification Document

8-Bit Fully Thermometric Differential Current-Steering DAC

| Document Number | RETYM-HYP-DAC-SPEC-001 |
| --- | --- |
| Version | 0.5 (Draft) |
| Date | 2026-10-18 |
| Author | [Author Name] |
| Status | DRAFT |

# Revision History

| Rev | Date | Author | Description |
| --- | --- | --- | --- |
| 0.1 | 2025-07-01 | [Author] | Initial draft – skeleton/template |
| 0.2 | 2026-07-20 | [Author] | Updated to 9-bit (511 codes); added INL/DNL formulas (sec 4.2) |
| 0.3 | 2026-10-18 | [Author] | 8-bit (255 units); spec in voltages: 500 mVpp diff FS, VCM 350 mV; DNL ±0.3 LSB incl. VCO error; sigma targets filled |
| 0.4 | 2026-10-18 | [Author] | Removed mismatch targets; added R_L, CM tol, gain error, 2 supplies, power split, settling (2τ), update rate, glitch, temp, C_L, VCO DNL share |
| 0.5 | 2026-10-18 | [Author] | FS = 1 Vpp diff (LSB 3.922 mV); internal currents I_FS 10 mA / I_LSB 39.2 µA; update-rate and glitch explanations in notes |

# Table of Contents

[Update field to generate TOC in Word: Ctrl+A → F9]

# 1. Introduction

## 1.1 Purpose

This document defines the specification for the Digital-to-Analog Converter (DAC) used in the HYP Gen1 Clock and Data Recovery (CDR) system. The DAC converts the digital loop filter output into an analog control voltage/current for the VCO/CCO.

## 1.2 Scope

This spec covers the DAC architecture, electrical performance targets, digital interface definition, timing requirements, and test/characterization plan.

## 1.3 References

| Ref # | Document | Description |
| --- | --- | --- |
| 1 | [CDR System Spec] | HYP Gen1 CDR top-level system specification |
| 2 | [Process PDK] | Foundry design manual and device models |
| 3 | [Layout Guidelines] | Current source matching layout rules |

## 1.4 Definitions & Abbreviations

| Term | Definition |
| --- | --- |
| CDR | Clock and Data Recovery |
| DAC | Digital-to-Analog Converter |
| DNL | Differential Non-Linearity |
| INL | Integral Non-Linearity |
| LSB | Least Significant Bit |
| VCO | Voltage Controlled Oscillator |
| CCO | Current Controlled Oscillator |
| BBPD | Bang-Bang Phase Detector |
| FLL | Frequency-Locked Loop |

# 2. System Overview

## 2.1 CDR Architecture Context

[Block diagram showing CDR loop: PD → Digital Loop Filter → DAC → VCO/CCO → ...]

The DAC resides between the digital loop filter and the analog VCO/CCO. It must translate digital frequency/phase correction commands into a precise analog control signal with minimal latency and noise.

## 2.2 DAC Role in CDR Loop

[Describe the DAC's role in loop dynamics, bandwidth, and jitter.]

## 2.3 Top-Level Block Diagram

[Insert DAC top-level block diagram here]

# 3. Architecture

## 3.1 DAC Topology

Topology: 8-bit fully thermometric, differential current-steering DAC.
Number of unit elements: N = 2^8 - 1 = 255 unit current sources.
Number of codes: 256 (0 … 255).

## 3.2 Unit Current Source Design

[Describe unit cell: transistor type, dimensions, cascode, etc.]

## 3.3 Switching Architecture

[Describe current-steering switch pair, return-to-zero options, etc.]

## 3.4 Bias Generation

[Describe bias current reference, distribution, and filtering.]

## 3.5 Output Stage

[Describe output termination, load, compliance voltage range.]

# 4. Electrical Specifications

## 4.1 Key Performance Parameters

| Parameter | Symbol | Min | Typ | Max | Unit | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Resolution | B | — | 8 | — | bits | Fully thermometric |
| Number of codes | — | — | 256 | — | — | 0 … 255 (255 unit steps) |
| Full-scale output (diff) | V_FS |  | 1000 |  | mVpp | Differential peak-to-peak (±500 mV) |
| LSB voltage (diff) | V_LSB |  | 3.922 |  | mV | V_FS / 255 |
| Output common-mode | V_CM | 332.5 | 350 | 367.5 | mV | ±5% – self spec |
| Load resistance | R_L |  | 50 |  | Ω | Per side |
| Full-scale current | I_FS |  | 10 |  | mA | Internal spec; V_FS = 2·I_FS·R_L |
| LSB (unit) current | I_LSB |  | 39.2 |  | µA | Internal spec; I_FS / 255 |
| Full-scale (gain) error | GE | -5 |  | +5 | % | Self spec |
| DNL | DNL | -0.3 |  | +0.3 | LSB | Including VCO error |
| DNL budget – VCO share | DNL_VCO |  | TBD |  | LSB | OPEN – part of the ±0.3 LSB, not known yet |
| INL | INL |  | TBD |  | LSB | Report only |
| Supply voltage (high) | VDD_1V2 | 1.14 | 1.20 | 1.26 | V |  |
| Supply voltage (low) | VDD_0V9 | 0.85 | 0.90 | 0.95 | V |  |
| Power – 1.2 V supply | P_1V2 |  | TBD |  | mW |  |
| Power – 0.9 V supply | P_0V9 |  | TBD |  | mW |  |
| Settling time (1 LSB) | t_set_LSB |  | TBD |  | ps | To 2τ = 86.5% of final value |
| Settling time (full scale) | t_set_FS |  | TBD |  | ns | To 2τ = 86.5% of final value |
| Update rate (UP/DN) | f_update |  | TBD |  | GHz | How often the code can change. The DAC is fully thermometric: every UP/DN pulse turns exactly one unit on or off, so the output always moves by exactly 1 LSB. There is no binary-to-thermometer decoder, so no decode delay and no multi-bit switching. Max rate = 1 / (UP/DN-to-switch delay + 1-LSB settling to 2τ). A faster pulse arrives before the previous step has settled. |
| Glitch (1-LSB step) | G |  | TBD |  | mV·ps | Glitch energy = area (mV × ps) of the output error spike during a code step, caused by switch timing skew and clock/charge feed-through. In a binary DAC the worst glitch is at mid-code (many bits switch at once). Here only one unit switches per step, so the glitch is small and the same for every code. Measured on V_diff. |
| Output noise (integrated, diff) | V_n |  | TBD |  | µV_rms | Table to be provided |
| PSRR | PSRR |  | TBD |  | dB | Report only |
| Load capacitance | C_L |  | TBD |  | fF | VCO input – to be completed |
| Temperature | T | -10 | 50 / 85 | 110 | °C | Typ: 50 and 85 (most likely both) |

## 4.2 INL and DNL Definitions

For a fully thermometric differential current-steering DAC with N = 2^B - 1 unit elements (B = 8, N = 255). All quantities are differential output voltages: V_diff = V_outp - V_outn, V_FS = 1000 mVpp, V_LSB = V_FS / 255 ≈ 3.922 mV.

### 4.2.1 DNL (Differential Non-Linearity)

DNL measures the deviation of each code step from the ideal 1-LSB step size.

Definition (endpoint-fitted):

    DNL(k) = [V_diff(k+1) - V_diff(k)] / V_LSB_actual  -  1

where:
    V_LSB_actual = [V_diff(N) - V_diff(0)] / N

For a fully thermometric DAC, each code transition activates exactly one unit element. Therefore:

    DNL(k) = [ΔV_u(k) - ΔV_avg] / ΔV_avg

where ΔV_u(k) is the differential output step of the k-th unit element and ΔV_avg is the mean step of all N units.

Specification: |DNL| < 0.3 LSB, including VCO error (guarantees monotonicity)

### 4.2.2 INL (Integral Non-Linearity)

INL measures the cumulative deviation of the transfer function from the ideal straight line.

Definition (endpoint-fitted):

    INL(k) = [V_diff(k) - V_diff(0)] / V_LSB_actual  -  k

Equivalently, INL is the cumulative sum of DNL:

    INL(k) = Σ_{i=0}^{k-1} DNL(i)

### 4.2.3 Differential DAC: Finite Rout Effect

For a differential current-steering DAC, the first-order finite-Rout effect cancels between the two output branches, producing only a gain error (no INL):

    Gain Error = (N × R_L) / (2 × R_out)

Second-order INL from Rout variation across code:

    INL_diff_max ≈ (N² / 42) × Δg_ds × R_L   [LSBs]

where Δg_ds is the variation of output conductance across the output voltage swing.

### 4.2.4 Mismatch-Limited DNL (Statistical)

Given relative mismatch σ_rel = σ(I_unit)/I_unit, the probability that ALL N codes meet |DNL| < DNL_target:

    Yield = [2·Φ(DNL_target / σ_rel) - 1]^N

where Φ is the standard normal CDF.

Solving for required σ_rel at a given yield target Y:

    σ_rel = DNL_target / Φ⁻¹[(1 + Y^(1/N)) / 2]

## 4.3 Noise Budget

[Define noise contributors: thermal, flicker, supply, reference.]

## 4.4 Power Budget

[Breakdown of power per supply (1.2 V / 0.9 V) and per sub-block.]

# 5. Digital Interface

## 5.1 Interface Type

[Define interface: async pulse (UP/DN), synchronous parallel, or hybrid.
Reference system_questions.md Q1–Q9 for open decisions.]

CODE[7:0]: 8-bit code (256 codes, 0 … 255)

## 5.2 Signal List

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| UP | Input | 1 | Increment DAC code by 1 LSB |
| DN | Input | 1 | Decrement DAC code by 1 LSB |
| RESET_N | Input | 1 | Active-low async reset to initial code |
| CODE[7:0] | Output/Internal | 8 | Current DAC code (optional readback) |
| SAT_HI | Output | 1 | Saturation flag – code at maximum |
| SAT_LO | Output | 1 | Saturation flag – code at minimum |

## 5.3 Protocol & Timing Diagram

[Insert timing diagram for UP/DN pulse interface.]

• Minimum pulse width: TBD ps
• Minimum pulse spacing: TBD ps
• Setup/hold to internal clock (if sync): TBD ps

## 5.4 Reset & Power-Up Behavior

[Define initial DAC code after reset: mid-code (128), min (0), or programmable.
Reference system_questions.md Q4.]

## 5.5 Saturation Handling

[Define behavior at code 0 and 255: clip, flag, or both.
Reference system_questions.md Q5.]

## 5.6 Clock Domain & Synchronization

[Define clock domain relationship between digital loop filter and DAC.
Reference system_questions.md Q8.]

# 6. Timing Specifications

## 6.1 Latency Budget

| Path | Latency | Unit | Notes |
| --- | --- | --- | --- |
| UP/DN pulse to output voltage change |  | ps | Interface + decoder + switch |
| Reset to mid-code settled |  | ns |  |
| Code-to-code settling (1 LSB) |  | ps | To 2τ (86.5%) |
| Full-scale settling (0 → 255) |  | ns | To 2τ (86.5%) |

## 6.2 Update Rate

The DAC is fully thermometric: every UP/DN pulse turns exactly one unit element on or off, so the output always moves by exactly 1 LSB. There is no binary-to-thermometer decoder, so no decode delay and no multi-bit switching.

Max update rate = 1 / (UP/DN-to-switch delay + 1-LSB settling to 2τ).
Value: TBD.

## 6.3 Glitch Energy

Glitch energy = area (mV·ps) of the output error spike during a code step, caused by switch timing skew and clock/charge feed-through.

In a binary DAC the worst glitch is at mid-code (many bits switch at once). In this fully thermometric DAC only one unit switches per step, so the glitch is small and the same for every code.
Value: TBD (1-LSB step, differential).

# 7. Layout & Matching Considerations

## 7.1 Unit Cell Layout

[Common-centroid, interdigitated, or other arrangement.]

## 7.2 Dummy Elements

[Number and placement of dummy devices at array edges.]

## 7.3 Routing & Shielding

[Signal routing strategy, ground shielding, guard rings.]

## 7.4 Area Estimate

[Estimated die area: TBD µm × TBD µm.]

# 8. Test & Characterization Plan

## 8.1 Production Tests

| Test | Method | Spec Limit | Notes |
| --- | --- | --- | --- |
| DNL | Code-by-code ramp | < ±0.3 LSB | Including VCO error |
| INL | Cumulative DNL | Report only |  |
| Full-scale voltage | Codes 0 and 255, measure V_diff | 1000 mVpp diff ±5% | Self spec |
| Common-mode voltage | Mid-code, measure V_CM | 350 mV ±5% | Self spec |
| Power consumption | Measure supply current | TBD |  |
| Monotonicity | Verify no code reversal (DNL > -1) | Pass/Fail |  |

## 8.2 Characterization Tests

| Test | Conditions | Purpose |
| --- | --- | --- |
| DNL vs. temperature | –10°C to 110°C | Verify matching over temp |
| DNL vs. supply | 1.14–1.26 V and 0.85–0.95 V | Supply impact on linearity |
| Settling time | 1 LSB and full-scale step | Verify timing spec |
| Noise spectral density | Mid-code, measure output | Verify noise budget |
| Glitch energy | Mid-code transition | Verify glitch spec |

## 8.3 Bench Measurement Setup

[Describe test equipment, PCB requirements, measurement methodology.]

# 9. Open Items & TBDs

The following items require resolution (see doc/system_questions.md):

| Item # | Question | Owner | Status |
| --- | --- | --- | --- |
| 1 | Step size: ±1 LSB only or variable? |  | Open |
| 2 | Frequency acquisition mode (FLL) exists? |  | Open |
| 3 | Max UP/DN pulse rate? |  | Open |
| 4 | Initial DAC code on reset? |  | Open |
| 5 | Saturation behavior (clip/flag/both)? |  | Open |
| 6 | UP/DN mutual exclusivity guaranteed? |  | Open |
| 7 | Code readback needed? |  | Open |
| 8 | Clock domain relationship? |  | Open |
| 9 | Supply / IO voltage levels? |  | Closed – 1.2 V and 0.9 V |
| 10 | VCO share of the ±0.3 LSB DNL budget? |  | Open |
| 11 | VCO input load capacitance (C_L)? |  | Open |
| 12 | Output noise table |  | Open |